| Flight #: 1 – FIRST TEST FLIGHT |  |  |  |
| --- | --- | --- | --- |
| Pilot: |  | Date: | Time: |
| Objectives: - Validate Engine Reliability - Explore Flight Control Characteristics |  |  |  |
| Check | Action |  |  |
|  | # DEPARTURE |  |  |
|  | Do not use flaps |  |  |
|  | Do not change throttle settings, mixture, or fuel tanks |  |  |
|  | Remain above the airport |  |  |
|  | Climb out at 100 MPH |  |  |
|  | Climb to 2000’ MSL and level off |  |  |
|  | # CRUISE |  |  |
|  | Limit prop RPM to 2300 |  |  |
|  | Check engine gauges especially CHT and Oil Temp |  |  |
|  | Trim hands off level flight |  |  |
|  | Yaw rudder left and right 5 degrees |  |  |
|  | 360 degree clearing turns (10 degrees bank) |  |  |
|  | 360 degree clearing turns (20 degrees bank) |  |  |

NOTES

| Check | Action |
| --- | --- |
|  | # SLOW FLIGHT |
|  | Climb to 4000 feet |
|  | Slowly decrease speed to 60 MPH – maintain altitude |
|  | 360 degree clearing turns at 20 degrees bank |
|  | Keep ball centered using rudder |
|  | Increase speed to 90 MPH |
|  | Apply 1st notch of flaps |
|  | Slowly decrease speed to 60 MPH – maintain altitude |
|  | 360 degree clearing turns at 20 degrees bank |
|  | Keep ball centered using rudder |
|  | Apply 2nd notch of flaps |
|  | Decrease speed to 55 mph |
|  | 360 degree clearing turns at 20 degrees bank |
|  | Increase speed to 65 while raising flaps |
|  | Check engine instruments |
|  | Speed back up to cruise speed (2300 RPM) |

NOTES

| Check | Action |
| --- | --- |
|  | # LANDING |
|  | Use checklists |
|  | Fly downwind pattern at 85 MPH |
|  | Fly Final Approach at 60 MPH |
|  | USE A MAXIMUM OF 25 DEGREES FLAPS |
|  | Taxi back and “Grin” |
|  | # POST FLIGHT |
|  | Prepare corrective action list |
|  | Record fuel and oil consumption |
|  | Pull cowlings and inspect engine carefully |
|  | Inspect airframe carefully |

NOTES

| Flight #: 2 – CONFIRM FIRST FLIGHT RESULTS |  |  |  |
| --- | --- | --- | --- |
| Pilot: |  | Date: | Time: |
| Objectives: - Re-affirm the first flight findings |  |  |  |
| Check | Action |  |  |
|  | # DEPARTURE |  |  |
|  | Do not use flaps |  |  |
|  | Do not change throttle settings, mixture, or fuel tanks |  |  |
|  | Remain above the airport |  |  |
|  | Climb out at 100 MPH |  |  |
|  | Climb to 2000’ MSL and level off |  |  |
|  | # CRUISE |  |  |
|  | Limit prop RPM to 2300 |  |  |
|  | Check engine gauges especially CHT and Oil Temp |  |  |
|  | Trim hands off level flight |  |  |
|  | Yaw rudder left and right 5 degrees |  |  |
|  | 360 degree clearing turns (10 degrees bank) |  |  |
|  | 360 degree clearing turns (20 degrees bank) |  |  |

NOTES

| Check | Action |
| --- | --- |
|  | # SLOW FLIGHT |
|  | Climb to 4000 feet |
|  | Slowly decrease speed to 60 MPH – maintain altitude |
|  | 360 degree clearing turns at 20 degrees bank |
|  | Keep ball centered using rudder |
|  | Increase speed to 85 MPH |
|  | Apply 2 notches of flaps (25 Degrees) |
|  | Slowly decrease speed to 55 MPH – maintain altitude |
|  | 360 degree clearing turns at 20 degrees bank |
|  | Keep ball centered using rudder |
|  | Apply 40 degrees of flaps - maintain 55 MPH |
|  | 360 turns |
|  | Add power, climb like doing a go-around & raise flaps |
|  | Check engine instruments |
|  | Speed back up to cruise speed |

NOTES

| Check | Action |
| --- | --- |
|  | # POST FLIGHT |
|  | Prepare corrective action list |
|  | Record fuel and oil consumption |
|  | Pull cowlings and inspect engine carefully |
|  | Inspect airframe carefully |

NOTES

| Flight #: 3 – VALIDATE ENGINE RELIABILITY |  |  |  |
| --- | --- | --- | --- |
| Pilot: |  | Date: | Time: |
| Objectives: - Validate that actions affecting engine operation function properly |  |  |  |
| Check | Action |  |  |
|  | # DEPARTURE |  |  |
|  | Do not use flaps |  |  |
|  | Do not change throttle settings, mixture, or fuel tanks |  |  |
|  | Remain above the airport |  |  |
|  | Climb out at 100 MPH |  |  |
|  | Climb to 6000’ MSL and level off |  |  |
|  | # CRUISE |  |  |
|  | Limit prop RPM to 2300 |  |  |
|  | Check engine gauges especially CHT and Oil Temp |  |  |
|  | Trim hands off level flight |  |  |
|  | Apply carb heat and note changes |  |  |
|  | Lean engine and note changes |  |  |
|  | Switch fuel tanks and note changes |  |  |

NOTES

| Check | Action |
| --- | --- |
|  | Record engine pressures and temperatures |
|  | # POST FLIGHT |
|  | Prepare corrective action list |
|  | Record fuel and oil consumption |
|  | Pull cowlings and inspect engine carefully |
|  | Inspect airframe carefully |

NOTES

| Flight #: 4 – SLOW FLIGHT TEST |  |  |  |
| --- | --- | --- | --- |
| Pilot: |  | Date: | Time: |
| Objectives: - Become familiar with slow flight handling characteristics |  |  |  |
| Check | Action |  |  |
|  | # DEPARTURE |  |  |
|  | Do not use flaps |  |  |
|  | Climb out at 100 MPH |  |  |
|  | Climb to 5000’ MSL and level off |  |  |
|  | # CRUISE |  |  |
|  | Limit prop RPM to 2300 |  |  |
|  | Check engine gauges especially CHT and Oil Temp |  |  |
|  | Trim hands off level flight |  |  |
|  | Perform (2) 360 degree clearing turns |  |  |
|  | Slow to 60 MPH |  |  |
|  | Feel untrimmed, then trim and maintain altitude |  |  |
|  | 360 turn left, then 360 right, shallow bank |  |  |
|  | Check CHTs & Oil Temp |  |  |
|  | Slow to 55 MPH |  |  |

NOTES

| Check | Action |
| --- | --- |
|  | # Trim & Maintain Altitude |
|  | # 360 turn left, 360 turn right, shallow bank |
|  | Check CHTs and Oil Temp |
|  | Slow to 50 MPH |
|  | Trim & maintain altitude |
|  | 360 turn left, 360 turn right, shallow bank |
|  | 10 degrees flaps, maintain airspeed, 360 left, 360 right |
|  | 25 degrees flaps, maintain airspeed, 360 left, 360 right |
|  | 40 degrees flaps, maintain airspeed, 360 left, 360 right |
|  | # 50 degrees flaps, maintain airspeed, 360 left, 360 right |
|  | # POST FLIGHT |
|  | Prepare corrective action list |
|  | Record fuel and oil consumption |

NOTES

| Flight #: 5 – CLIMBS AND DESCENTS |  |  |  |
| --- | --- | --- | --- |
| Pilot: |  | Date: | Time: |
| Objectives: - Monitor engine performance during climbs and descents |  |  |  |
| Check | Action |  |  |
|  | # DEPARTURE |  |  |
|  | Do not use flaps |  |  |
|  | Climb out at 100 MPH |  |  |
|  | Climb to 2000’ MSL and level off |  |  |
|  | # CRUISE |  |  |
|  | Limit prop RPM to 2300 |  |  |
|  | Check engine gauges especially CHT and Oil Temp |  |  |
|  | Trim hands off level flight |  |  |
|  | 120 MPH climb for two minutes – full power |  |  |
|  | Record engine temperatures and pressures – and OAT |  |  |
|  | Stabilize temperatures |  |  |
|  | 110 MPH climb for two minutes – full power |  |  |
|  | Record engine temperatures and pressures – and OAT |  |  |
|  | Stabilize temperatures |  |  |

NOTES

To obtain this test flight data you must use common sense.  Don't cook your engine!  Don't shock cool your engine!  Elements of this testing might be done in conjunction with other test flights during several different flights to avoid overheating or shock cooling your engine.

| Check | Action |
| --- | --- |
|  | # Moderate power descent to 2000 ‘ – do not exceed 175 MPH |
|  | 100 MPH climb for two minutes – full power |
|  | Record engine temperatures and pressures – and OAT |
|  | Stabilize temperatures |
|  | 90 MPH climb for two minutes – full power |
|  | Record engine temperatures and pressures – and OAT |
|  | Stabilize temperatures |
|  | # POST FLIGHT |
|  | Prepare corrective action list |
|  | Record fuel and oil consumption |

NOTES

| Flight #: 6 – AIRSPEED IN-FLIGHT ACCURACY CHECK |  |  |  |
| --- | --- | --- | --- |
| Pilot: |  | Date: | Time: |
| Objectives: - Determine accuracy of the airspeed indicator throughout a range of airspeeds |  |  |  |
| Check | Action |  |  |
|  | Determine altitudes at which you desire airspeed data. |  |  |
|  | # DEPARTURE |  |  |
|  | Do not use flaps |  |  |
|  | Climb out at 100 MPH |  |  |
|  | Climb to 2000’ MSL and level off |  |  |
|  | # CRUISE |  |  |
|  | Limit prop RPM to 1700 |  |  |
|  | Check engine gauges especially CHT and Oil Temp |  |  |
|  | Trim hands off level flight |  |  |
|  | Set Mixture |  |  |

NOTES

When you fly the three ground tracks for each power setting the IAS should be exactly the same.  If not, make sure you're at the exact same altitude and just be patient --- sometimes it takes a few minutes for the airspeed to settle after a turn and a few burbbles.

| Check | Action |
| --- | --- |
|  | # 1700 RPM, constant altitude - 5500 MSL |
|  | - Record OAT |
|  | - Record MAP |
|  | - Fly ground track 360 |
|  | - Record IAS from airspeed indicator |
|  | - Record ground speed from GPS |
|  | - Fly ground track 180 |
|  | - Record IAS from airspeed indicator |
|  | - Record ground speed from GPS |
|  | - Fly ground track 90 |
|  | - Record IAS from airspeed indicator |
|  | - Record ground speed from GPS |
|  | - Record ground track from GPS |

NOTES

Altitude:	5500 MSL

Throttle:	1700 RPM

|  | 360 | 180 | 90 |
| --- | --- | --- | --- |
| OAT |  |  |  |
| MAP |  |  |  |
| IAS |  |  |  |
| Ground Speed |  |  |  |

| Check | Action |
| --- | --- |
|  | # Set power 2000 RPM, constant altitude - 5500 MSL |
|  | - Reset Mixture |
|  | - Record MAP |
|  | - Fly ground track 360 |
|  | - Record IAS from airspeed indicator |
|  | - Record ground speed from GPS |
|  | - Fly ground track 180 |
|  | - Record IAS from airspeed indicator |
|  | - Record ground speed from GPS |
|  | - Fly ground track 90 |
|  | - Record IAS from airspeed indicator |
|  | - Record ground speed from GPS |

NOTES

Altitude:	5500 MSL

Throttle:	2000 RPM

|  | 360 | 180 | 90 |
| --- | --- | --- | --- |
| MAP |  |  |  |
| IAS |  |  |  |
| Ground Speed |  |  |  |

| Check | Action |
| --- | --- |
|  | # Set power 2400 RPM, constant altitude - 5500 MSL |
|  | - Reset Mixture |
|  | - Record MAP |
|  | - Fly ground track 360 |
|  | - Record IAS from airspeed indicator |
|  | - Record ground speed from GPS |
|  | - Fly ground track 180 |
|  | - Record IAS from airspeed indicator |
|  | - Record ground speed from GPS |
|  | - Fly ground track 90 |
|  | - Record IAS from airspeed indicator |
|  | - Record ground speed from GPS |

NOTES

Altitude:	5500 MSL

Throttle:	2400 RPM

|  | 360 | 120 | 240 |
| --- | --- | --- | --- |
| Manifold Pressure |  |  |  |
| IAS |  |  |  |
| Ground Speed |  |  |  |

| Check | Action |
| --- | --- |
|  | # Set power 2600 RPM, constant altitude - 5500 MSL |
|  | - Reset Mixture |
|  | - Record MAP |
|  | - Fly ground track 360 |
|  | - Record IAS from airspeed indicator |
|  | - Record ground speed from GPS |
|  | - Fly ground track 180 |
|  | - Record IAS from airspeed indicator |
|  | - Record ground speed from GPS |
|  | - Fly ground track 90 |
|  | - Record IAS from airspeed indicator |
|  | - Record ground speed from GPS |

NOTES

Altitude:	5500 MSL

Throttle:	2600 RPM

|  | 360 | 120 | 240 |
| --- | --- | --- | --- |
| Manifold Pressure |  |  |  |
| IAS |  |  |  |
| Ground Speed |  |  |  |

| Check | Action |
| --- | --- |
|  | # POST FLIGHT |
|  | Prepare corrective action list |
|  | Record fuel and oil consumption |
|  | Calculate True Airspeeds using True Airspeed Calculator |
|  | Update Aircraft Operations Manual |

NOTES

What you're testing is the accuracy of your airspeed indicator.  Consider doing this test at close to stall speeds w/ & w/out flaps to get an idea of your TAS for stall speed.

| Flight #: 7 – STALLS |  |  |  |
| --- | --- | --- | --- |
| Pilot: |  | Date: | Time: |
| Objectives: - Determine actual stall speeds in landing and takeoff configuration |  |  |  |
| Check | Action |  |  |
|  | # PRE-FLIGHT |  |  |
|  | Fill fuel tanks to full |  |  |
|  | # DEPARTURE |  |  |
|  | Do not use flaps |  |  |
|  | Climb out at 100 MPH |  |  |
|  | Climb to 5000’ MSL and level off |  |  |
|  | # CRUISE |  |  |
|  | Limit prop RPM to 2200 |  |  |
|  | Check engine gauges especially CHT and Oil Temp |  |  |
|  | Trim hands off level flight |  |  |
|  | # 360 degree clearing turns |  |  |

NOTES

| Check | Action |
| --- | --- |
|  | # POWER OFF STALLS |
|  | No flaps |
|  | Slowly decelerate while maintaining altitude |
|  | Keep ball centered with rudder |
|  | Note stall speed |
|  | Recover altitude and speed |
|  | Slow down to 80 MPH |
|  | Apply 25 degrees flaps |
|  | Slowly decelerate while maintaining altitude |
|  | Keep ball centered with rudder |
|  | Note stall speed |
|  | Retract flaps |
|  | Recover altitude and speed |

NOTE SPEED IAS (MPH)

POWER-OFF STALL, NO FLAP:

POWER-OFF STALL, 25 DEGREE FLAPS:

| Check | Action |
| --- | --- |
|  | Slow down to 65 MPH |
|  | Apply 40 degree flaps |
|  | Slowly decelerate while maintaining altitude |
|  | Keep ball centered with rudder |
|  | Note stall speed |
|  | Retract flaps |
|  | Recover altitude and speed |
|  | # Repeat above but with 50 degrees flaps |
|  | # POWER ON STALLS |
|  | Set power to 2200 RPM |
|  | Slowly pull back elevator |
|  | Keep ball centered with rudder |
|  | Note stall speed |
|  | Recover altitude and speed |

NOTES  SPEED IAS (MPH)

POWER-OFF STALL, 40 DEGREES FLAPS:

POWER-OFF STALL, 50 DEGREES FLAPS:

POWER-ON STALL, 2200 RPM, NO FLAP:

| Check | Action |
| --- | --- |
|  | Full power |
|  | Slowly pull back elevator |
|  | Keep ball centered with rudder |
|  | Note stall speed |
|  | Recover altitude and speed |
|  | # POST FLIGHT |
|  | Prepare corrective action list |
|  | Record fuel and oil consumption |
|  | Update POH with actual stall speeds |

NOTES  SPEED IAS (MPH)

POWER-ON STALL, FULL POWER:

| Flight #: 8 – CLIMB SPEEDS |  |  |  |
| --- | --- | --- | --- |
| Pilot: |  | Date: | Time: |
| Objectives: - Establish best rate of climb speed (Vy) - Establish best angle of climb speed (Vx) - Establish best glide rates - Learn to visualize power-off glide descent rate |  |  |  |
| Check | Action |  |  |
|  | # PRE-FLIGHT |  |  |
|  | Bring small stopwatch/timer |  |  |
|  | # DEPARTURE |  |  |
|  | Do not use flaps |  |  |
|  | Climb out at 100 MPH |  |  |
|  | Climb to 1500’ MSL and level off |  |  |
|  | # CRUISE |  |  |
|  | Limit prop RPM to 2200 |  |  |
|  | Check engine gauges especially CHT and Oil Temp |  |  |
|  | Trim hands off level flight, Full rich mixture |  |  |
|  | # Do 2 clearing turns |  |  |

NOTES

As mentioned before, use common sense.  Don't cook your engine and don't shock cool your engine.  These tests might be best done over several flights in conjunction with other tests.

| Check | Action |
| --- | --- |
|  | # CLIMB TEST#1 |
|  | Establish 110 MPH climb - Trim hands off |
|  | Begin 1 minute timer as you pass thru 2000 MSL |
|  | At end of 1 minute, record altitude |
|  | Ending Altitude= FPM= |
|  | Trim Level, Cool Engine |
|  | GLIDE TEST #1 |
|  | Descend at 110 MPH - Trim hands off |
|  | Record descent rate from VSI |
|  | Perform a 90 degree turn @ 15 deg. bank and record altitude lost |
|  | Perform 180 degree turn @ 15 deg. bank and record altitude lost |
|  | Perform 360 degree turn @ 15 deg. bank and record altitude lost |
|  | CLIMB TEST#2 |
|  | Establish 100 MPH climb - Trim hands off |
|  | Begin 1 minute timer as we pass thru 2000 MSL |
|  | At end of 1 minute, record altitude |
|  | Ending Altitude= FPM= |
|  | # Trim Level, Cool Engine |
|  | Descend at 100 MPH, Trim hands off, Record Rate |
|  | Perform & record altitude lost in 90, 180 & 360 turns |

NOTES

| IAS | Climbed to: | Climb Rate (FPM) | Descent Rate (FPM) | 90 Turn | 180 Turn | 360 Turn |
| --- | --- | --- | --- | --- | --- | --- |
| 110 |  |  |  |  |  |  |
| 100 |  |  |  |  |  |  |

| Check | Action |
| --- | --- |
|  | # CLIMB TEST#3 |
|  | Establish 90 MPH climb - Trim hands off |
|  | Begin 1 minute timer as we pass thru 2000 MSL |
|  | At end of 1 minute, record altitude |
|  | Ending Altitude= FPM= |
|  | # Descend to 1500 MSL |
|  | Trim Level, Cool Engine |
|  | Descend at 90 MPH, Trim hands off, Record Rate |
|  | Perform & record altitude lost in 90, 180 & 360 turns |
|  | CLIMB TEST#4 |
|  | Establish 80 MPH climb - Trim hands off |
|  | Begin 1 minute timer as we pass thru 2000 MSL |
|  | At end of 1 minute, record altitude |
|  | Ending Altitude= FPM= |
|  | # Descend to 1500 MSL |
|  | Trim Level, Cool Engine |
|  | Descend at 80 MPH, Trim hands off, Record Rate |
|  | Perform & record altitude lost in 90, 180 & 360 turns |

NOTES

| IAS | Climbed to: |  | Climb Rate (FPM) |  | Descent Rate (FPM) |  | 90 turn |  | 180 turn |  | 360 turn |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 90 |  |  |  |  |  |  |  |  |  |  |  |  |
| 80 |  |  |  |  |  |  |  |  |  |  |  |  |

| Check | Action |
| --- | --- |
|  | # CLIMB TEST#5 |
|  | Establish 70 MPH climb - Trim hands off |
|  | Begin 1 minute timer as we pass thru 2000 MSL |
|  | At end of 1 minute, record altitude |
|  | Ending Altitude= FPM= |
|  | # Descend to 1500 MSL |
|  | Trim Level, Cool Engine |
|  | Descend at 70 MPH, Trim hands off, Record Rate |
|  | Perform & record altitude lost in 90, 180 & 360 turns |
|  | CLIMB TEST#6 |
|  | Establish 60 MPH climb - Trim hands off |
|  | Begin 1 minute timer as we pass thru 2000 MSL |
|  | At end of 1 minute, record altitude |
|  | Ending Altitude= FPM= |
|  | # Descend to 1500 MSL |
|  | Trim Level, Cool Engine |
|  | Descend at 60 MPH, Trim hands off, Record Rate |
|  | Perform & record altitude lost in 90, 180 & 360 turns |

NOTES

| IAS | Climbed to: |  | Climb Rate (FPM) |  | Descent Rate (FPM) |  | 90 turn |  | 180 turn |  | 360 turn |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 70 |  |  |  |  |  |  |  |  |  |  |  |  |
| 60 |  |  |  |  |  |  |  |  |  |  |  |  |

| Check | Action |
| --- | --- |
|  | LANDING |
|  | Use checklists |
|  | Fly pattern at 80 MPH |
|  | Taxi back and "Grin" |
|  | POST FLIGHT |
|  | Prepare corrective action list |
|  | Record fuel and oil consumption |
|  | Use graph to compute Vy and Vx |
|  | Use graph to compute best glide speed |

NOTES

| Flight #: 9 – STABILITY AND CONTROL CHECKS |  |  |  |
| --- | --- | --- | --- |
| Pilot: |  | Date: | Time: |
| Objectives: - Determine longitudinal stability - Determine lateral-directional stability - Determine spiral stability |  |  |  |
| Check | Action |  |  |
|  | # PRE-FLIGHT |  |  |
|  | These tests cannot be accomplished until any necessary trim tabs have been installed so the aircraft can be flown hands off |  |  |
|  | # DEPARTURE |  |  |
|  | Do not use flaps |  |  |
|  | Climb out at 110 MPH |  |  |
|  | Climb to 6000’ MSL and level off |  |  |
|  | # CRUISE |  |  |
|  | Limit prop RPM to 2300 |  |  |
|  | Check engine gauges especially CHT and Oil Temp |  |  |
|  | Trim hands off level flight |  |  |

NOTES

| Check | Action |
| --- | --- |
|  | LONGITUDINAL STABILITY TEST |
|  | # Record airspeed at 2300 RPM (Airspeed ___ = A) |
|  | Lightly pull on stick to reduce airspeed by 10% |
|  | New airspeed: A-10%= ___ = B |
|  | Does the acft. require continued pull force to maintain the new airspeed? |
|  | If yes: Pull stick to reduce airspeed to A-20% =___= C |
|  | # Does acft. require still more pull force to maintain airspeed C? |
|  | If yes: N57EN has POSITIVE static stability |
|  | If no to either B or C airspeed, N57EN has NEUTRAL static stability |
|  | If N57EN requires a push force for B or C airspeeds, then N57EN has NEGATIVE static stability |
|  | ---- repeat test using a PUSH test instead of PULL test |
|  | TEST FOR POSITIVE DYNAMIC LONGITUDINAL STABILITY (SHORT PERIOD) |
|  | Trim for cruise @ 2300 RPM |
|  | Push nose down 5 degrees, then up to level attitude |
|  | As attitude reaches level, release stick |
|  | # If N57EN briefly oscillates about the trim attitude before settling at trim attitude then N57EN has POSITIVE DYNAMIC LONGITUDINAL STABILITY (SHORT PERIOD) |

NOTES

| Check | Action |
| --- | --- |
|  | TEST FOR POSITIVE DYNAMIC LONGITUDINAL STABILITY (LONG PERIOD) |
|  | # Trim for cruise @ 2300 RPM, Record Airspeed ___ = A |
|  | Pull stick for A - 5 MPH and release stick |
|  | Expect N57EN to oscillate about the trim speed before in dampens out |
|  | If amplitude INCREASES with time = NEGATIVE DLS |
|  | If amplitude CONTINUES to oscillate = NEUTRAL DLS |
|  | # If N57EN returns to cruise trim & speed = POSITIVE DLS |
|  | TEST FOR LATERAL/DIRECTIONAL CONTROL STABILITY |
|  | Set low cruise speed (BELOW MANEUVERING SPEED) & trim |
|  | Slowly enter a sideslip until either full rudder or full aileron deflection |
|  | # Release aileron while holding full rudder |
|  | ---> low wing should raise to level |
|  | TEST FOR STATIC DIRECTIONAL STABILITY |
|  | Set low cruise speed (BELOW MANEUVERING SPEED) & trim |
|  | # Slowly yaw N57EN with rudder while keeping acft level with aileron ----- release rudder |
|  | N57EN should return to no yaw condition |

NOTES

| Check | Action |
| --- | --- |
|  | TEST SPIRAL STABILITY |
|  | # (This will demonstrate the aircraft's tendency to raise the low wing when controls are released in a bank) |
|  | Bank 15 to 20 degrees and release controls |
|  | ---> If bank angle DECREASES = POSITIVE SS |
|  | ---> If bank angle STAYS THE SAME = NEUTRAL SS |
|  | ---> If bank angle INCREASES = NEGATIVE SS |
|  | # LANDING |
|  | Use checklists |
|  | Fly pattern at 85 MPH |
|  | Taxi back and “Smile” |
|  | # POST FLIGHT |
|  | Prepare corrective action list |
|  | Record fuel and oil consumption |

NOTES

| Flight #: 10 – ACCELERATED STALLS |  |  |  |
| --- | --- | --- | --- |
| Pilot: |  | Date: | Time: |
| Objectives: - Further explore stall characteristics of the aircraft |  |  |  |
| Check | Action |  |  |
|  | # PRE-FLIGHT |  |  |
|  | Consider wearing a parachute & practice egress |  |  |
|  | # DEPARTURE |  |  |
|  | Do not use flaps |  |  |
|  | Climb out at 110 MPH |  |  |
|  | Climb to 6,000’ AGL and level off |  |  |
|  | # CRUISE |  |  |
|  | Limit Airspeed to MANEUVERING SPEED |  |  |
|  | Check engine gauges especially CHT and Oil Temp |  |  |
|  | Trim hands off level flight |  |  |

NOTES

| Check | Action |
| --- | --- |
|  | # ACCELERATED STALL TEST |
|  | Hold 15 degrees bank and slow the aircraft until stall |
|  | ---> Airspeed at stall with 15 degrees bank = ___ |
|  | ---> Airspeed at stall with 30 degrees bank = ___ |
|  | ---> Airspeed at stall with 45 degrees bank = ___ |
|  | ---> Airspeed at stall with 60 degrees bank = ___ (2g) |
|  | # LANDING |
|  | Use checklists |
|  | Fly pattern at 85 MPH |
|  | Taxi back and “Smile” |
|  | # POST FLIGHT |
|  | Prepare corrective action list |
|  | Record fuel and oil consumption |

NOTES

| Flight #: 11a – EXPLORE WEIGHT & BALANCE LIMITS |  |  |  |
| --- | --- | --- | --- |
| Pilot: |  | Date: | Time: |
| Objectives: Determine affect of change to aft balance and progressively increasing weights to establish maximum weight - Pilot (me) plus 250 lb. passenger & max. fuel |  |  |  |
| Check | Action |  |  |
|  | # PRE-FLIGHT |  |  |
|  | Carefully weigh and secure ballast |  |  |
|  | # Compute & record new weight & balance |  |  |
|  | # DEPARTURE |  |  |
|  | Note: 10% flaps can help lift tail as aft weight is added. |  |  |
|  | Climb out at 110 MPH |  |  |
|  | Climb to 6000’ MSL and level off |  |  |
|  | Record climb performance: FPM = ____ |  |  |
|  | # CRUISE |  |  |
|  | Limit prop RPM to 2300 |  |  |
|  | Check engine gauges especially CHT and Oil Temp |  |  |
|  | Trim hands off level flight |  |  |
|  | # SLOW FLIGHT & STALLS |  |  |
|  | 360 turns, slow flight & power off stall w/ no flaps, 1/2 flaps, full flaps, recover |  |  |
|  | Execute power-on stalls @ 2200 RPM |  |  |

NOTES

Salt bags work well as ballast.  Make sure they are securely belted in.

| Check | Action |
| --- | --- |
|  | STABILITY & CONTROL CHECKS |
|  | # Longitudinal Stability: Record Airspeed @ 2200 RPM (A) |
|  | Pull to reduce to A-10%=B, require continued pull for B? Yes=Positive LS |
|  | Do push test |
|  | Lateral/Directional Control Stability: Sideslip |
|  | Release Aileron (keep rudder), Do wings level? |
|  | # Static Directional Stability: Yaw w/ level wings, release rudder. Return to no yaw? |
|  | Spiral Stability: Bank 15 deg., release. Return to level? |
|  | ACCELERATED STALLS |
|  | 15 Degree bank, slow until stalls: Repeat w/ 30, 45, 60 |
|  | DESCENT RATE |
|  | Descend @ 90 MPH, record rate. Record loss of elevation w/ 90, 180, 360 degree turns |
|  | # POST FLIGHT |
|  | Prepare corrective action list |
|  | Record fuel and oil consumption |

NOTES

DO NOT EXCEED GROSS WEIGHT.  STAY WITHING THE FORE/AFT CG LIMITS.

| Flight #: 11b – EXPLORE WEIGHT & BALANCE LIMITS |  |  |  |
| --- | --- | --- | --- |
| Pilot: |  | Date: | Time: |
| Objectives: Determine affect of change to aft balance and progressively increasing weights to establish maximum weight - Pilot (me) plus 250 lb. Passenger in front, Rear seat passenger 250 lb. & max. fuel |  |  |  |
| Check | Action |  |  |
|  | # PRE-FLIGHT |  |  |
|  | Carefully weigh and secure balast |  |  |
|  | # Compute & record new weight & balance |  |  |
|  | # DEPARTURE |  |  |
|  | Climb out at 110 MPH |  |  |
|  | Climb to 6000’ MSL and level off |  |  |
|  | Record climb performance: FPM = ____ |  |  |
|  | # CRUISE |  |  |
|  | Limit prop RPM to 2300 |  |  |
|  | Check engine gauges especially CHT and Oil Temp |  |  |
|  | Trim hands off level flight |  |  |
|  | # SLOW FLIGHT & STALLS |  |  |
|  | 360 turns, slow flight & power off stall w/ no flaps, 1/2 flaps, full flaps, recover |  |  |
|  | Execute power-on stalls @ 2200 RPM |  |  |

NOTES

| Check | Action |
| --- | --- |
|  | STABILITY & CONTROL CHECKS |
|  | # Longitudinal Stability: Record Airspeed @ 2200 RPM (A) |
|  | Pull to reduce to A-10%=B, require continued pull for B? Yes=Positive LS |
|  | Do push test |
|  | Lateral/Directional Control Stability: Sideslip |
|  | Release Aileron (keep rudder), Do wings level? |
|  | # Static Directional Stability: Yaw w/ level wings, release rudder. Return to no yaw? |
|  | Spiral Stability: Bank 15 deg., release. Return to level? |
|  | ACCELERATED STALLS |
|  | 15 Degree bank, slow until stalls: Repeat w/ 30, 45, 60 |
|  | DESCENT RATE |
|  | Descend @ 90 MPH, record rate. Record loss of elevation w/ 90, 180, 360 degree turns |
|  | # POST FLIGHT |
|  | Prepare corrective action list |
|  | Record fuel and oil consumption |

NOTES

| Flight #: 11c – EXPLORE WEIGHT & BALANCE LIMITS |  |  |  |
| --- | --- | --- | --- |
| Pilot: |  | Date: | Time: |
| Objectives: Determine affect of change to aft balance and progressively increasing weights to establish maximum weight - Pilot (me) plus 250 lb. Passenger on right, 200lb rear seat passenger, 130 lb baggage & max. fuel |  |  |  |
| Check | Action |  |  |
|  | # PRE-FLIGHT |  |  |
|  | Carefully weigh and secure balast |  |  |
|  | # Compute & record new weight & balance |  |  |
|  | # DEPARTURE |  |  |
|  | Climb out at 110 MPH |  |  |
|  | Climb to 6000’ MSL and level off |  |  |
|  | Record climb performance: FPM = ____ |  |  |
|  | # CRUISE |  |  |
|  | Limit prop RPM to 2300 |  |  |
|  | Check engine gauges especially CHT and Oil Temp |  |  |
|  | Trim hands off level flight |  |  |
|  | # SLOW FLIGHT & STALLS |  |  |
|  | 360 turns, slow flight & power off stall w/ no flaps, 1/2 flaps, full flaps, recover |  |  |
|  | Execute power-on stalls @ 2200 RPM |  |  |

NOTES

| Check | Action |
| --- | --- |
|  | STABILITY & CONTROL CHECKS |
|  | # Longitudinal Stability: Record Airspeed @ 2200 RPM (A) |
|  | Pull to reduce to A-10%=B, require continued pull for B? Yes=Positive LS |
|  | Do push test |
|  | Lateral/Directional Control Stability: Sideslip |
|  | Release Aileron (keep rudder), Do wings level? |
|  | # Static Directional Stability: Yaw w/ level wings, release rudder. Return to no yaw? |
|  | Spiral Stability: Bank 15 deg., release. Return to level? |
|  | ACCELERATED STALLS |
|  | 15 Degree bank, slow until stalls: Repeat w/ 30, 45, 60 |
|  | DESCENT RATE |
|  | Descend @ 90 MPH, record rate. Record loss of elevation w/ 90, 180, 360 degree turns |
|  | # LANDING |
|  | Use checklists |
|  | Fly pattern at 85 MPH |
|  | Taxi back and “Smile” |
|  | # POST FLIGHT |
|  | Prepare corrective action list |
|  | Record fuel and oil consumption |

NOTES

| Flight #: 12a – FUEL CONSUMPTION |  |  |  |
| --- | --- | --- | --- |
| Pilot: |  | Date: | Time: |
| Objectives: - Determine fuel burn during takeoff/climb/descent to/from 3500 feet - Determine fuel burn at various power settings at 3500 feet |  |  |  |
| Check | Action |  |  |
|  | # PRE-FLIGHT |  |  |
|  | Create GPS racetrack w/ 10 mile legs |  |  |
|  | Fill both tanks to full |  |  |
|  | # DEPARTURE |  |  |
|  | Climb out at 100 MPH USING RIGHT TANK |  |  |
|  | Climb to 3500’ MSL and level off |  |  |
|  | # CRUISE |  |  |
|  | Trim for cruise 2300 RPM, Record IAS ___ |  |  |
|  | Switch to LEFT TANK |  |  |
|  | Start Timer for 30 minutes, Fly racetrack |  |  |
|  | Record MAP, OAT, and everything else |  |  |

NOTES

| Check | Action |
| --- | --- |
|  | # LANDING |
|  | At end of 30 minutes, switch tanks & descend to land |
|  | Use checklists |
|  | Fly pattern at 85 MPH |
|  | Taxi back and “Smile” |
|  | # POST FLIGHT |
|  | Measure fuel in each tank & fill |
|  | Record fuel burned & compute consumption rate |
|  | Left tank fuel added (burned) x 2 = GPH @ 2400 RPM (XX% power) @ 3500 feet. |
|  | Right tank fuel added (burned) = fuel required to climb/descend to 3500 feet. |
|  | Prepare corrective action list |
|  | Record fuel and oil consumption |
|  | - Repeat at this altitude using 2500, 2600 and 2700 RPM |

NOTES

| Flight #: 12b – FUEL CONSUMPTION |  |  |  |
| --- | --- | --- | --- |
| Pilot: |  | Date: | Time: |
| Objectives: - Determine fuel burn during takeoff/climb/descent to/from 5500 feet - Determine fuel burn at various power settings at 3500 feet |  |  |  |
| Check | Action |  |  |
|  | # PRE-FLIGHT |  |  |
|  | Create GPS racetrack w/ 10 mile legs |  |  |
|  | Fill both tanks to full |  |  |
|  | # DEPARTURE |  |  |
|  | Climb out at 110 MPH USING RIGHT TANK |  |  |
|  | Climb to 5500’ MSL and level off |  |  |
|  | # CRUISE |  |  |
|  | Trim for cruise 2300 RPM, Record IAS ___ |  |  |
|  | Switch to LEFT TANK |  |  |
|  | Start Timer for 30 minutes, Fly racetrack |  |  |
|  | Record MAP, OAT, and everything else |  |  |

NOTES

| Check | Action |
| --- | --- |
|  | # LANDING |
|  | At end of 30 minutes, switch tanks & descend to land |
|  | Use checklists |
|  | Fly pattern at 85 MPH |
|  | Taxi back and “Smile” |
|  | # POST FLIGHT |
|  | Measure fuel in each tank & fill |
|  | Record fuel burned & compute consumption rate |
|  | Left tank fuel added (burned) x 2 = GPH @ 2400 RPM (XX% power) @ 5500 feet. |
|  | Right tank fuel added (burned) = fuel required to climb/descend to 5500 feet. |
|  | Prepare corrective action list |
|  | Record fuel and oil consumption |
|  | - Repeat at this altitude using 2500, 2600 and 2700 RPM |

NOTES

| Flight #: 12c – FUEL CONSUMPTION |  |  |  |
| --- | --- | --- | --- |
| Pilot: |  | Date: | Time: |
| Objectives: - Determine fuel burn during takeoff/climb/descent to/from 7500 feet - Determine fuel burn at various power settings at 7500 feet |  |  |  |
| Check | Action |  |  |
|  | # PRE-FLIGHT |  |  |
|  | Create GPS racetrack w/ 10 mile legs |  |  |
|  | Fill both tanks to full |  |  |
|  | # DEPARTURE |  |  |
|  | Climb out at 110 MPH USING RIGHT TANK |  |  |
|  | Climb to 7500’ MSL and level off |  |  |
|  | # CRUISE |  |  |
|  | Trim for cruise 2300 RPM, Record IAS ___ |  |  |
|  | Switch to LEFT TANK |  |  |
|  | Start Timer for 30 minutes, Fly racetrack |  |  |
|  | Record MAP, OAT, and everything else |  |  |

NOTES

| Check | Action |
| --- | --- |
|  | # LANDING |
|  | At end of 30 minutes, switch tanks & descend to land |
|  | Use checklists |
|  | Fly pattern at 85 MPH |
|  | Taxi back and “Smile” |
|  | # POST FLIGHT |
|  | Measure fuel in each tank & fill |
|  | Record fuel burned & compute consumption rate |
|  | Left tank fuel added (burned) x 2 = GPH @ 2400 RPM (XX% power) @ 7500 feet. |
|  | Right tank fuel added (burned) = fuel required to climb/descend to 7500 feet. |
|  | Prepare corrective action list |
|  | Record fuel and oil consumption |
|  | - Repeat at this altitude using 2500, 2600 and 2700 RPM |

NOTES

| Flight #: 12d – FUEL CONSUMPTION |  |  |  |
| --- | --- | --- | --- |
| Pilot: |  | Date: | Time: |
| Objectives: - Determine fuel burn during takeoff/climb/descent to/from 9500 feet - Determine fuel burn at various power settings at 3500 feet |  |  |  |
| Check | Action |  |  |
|  | # PRE-FLIGHT |  |  |
|  | Create GPS racetrack w/ 10 mile legs |  |  |
|  | Fill both tanks to full |  |  |
|  | # DEPARTURE |  |  |
|  | Climb out at 110 MPH USING RIGHT TANK |  |  |
|  | Climb to 9500’ MSL and level off |  |  |
|  | # CRUISE |  |  |
|  | Trim for cruise 2300 RPM, Record IAS ___ |  |  |
|  | Switch to LEFT TANK |  |  |
|  | Start Timer for 30 minutes, Fly racetrack |  |  |
|  | Record MAP, OAT, and everything else |  |  |

NOTES

| Check | Action |
| --- | --- |
|  | # LANDING |
|  | At end of 30 minutes, switch tanks & descend to land |
|  | Use checklists |
|  | Fly pattern at 85 MPH |
|  | Taxi back and “Smile” |
|  | # POST FLIGHT |
|  | Measure fuel in each tank & fill |
|  | Record fuel burned & compute consumption rate |
|  | Left tank fuel added (burned) x 2 = GPH @ 2400 RPM (XX% power) @ 9500 feet. |
|  | Right tank fuel added (burned) = fuel required to climb/descend to 9500 feet. |
|  | Prepare corrective action list |
|  | Record fuel and oil consumption |
|  | - Repeat at this altitude using 2500, 2600 and 2700 RPM |

NOTES

| Flight #: 12e – FUEL CONSUMPTION |  |  |  |
| --- | --- | --- | --- |
| Pilot: |  | Date: | Time: |
| Objectives: - Determine fuel burn during takeoff/climb/descent to/from 11,500 feet - Determine fuel burn at various power settings at 11,500 feet |  |  |  |
| Check | Action |  |  |
|  | # PRE-FLIGHT |  |  |
|  | Create GPS racetrack w/ 10 mile legs |  |  |
|  | Fill both tanks to full |  |  |
|  | # DEPARTURE |  |  |
|  | Climb out at 110 MPH USING RIGHT TANK |  |  |
|  | Climb to 11,500’ MSL and level off |  |  |
|  | # CRUISE |  |  |
|  | Trim for cruise 2300 RPM, Record IAS ___ |  |  |
|  | Switch to LEFT TANK |  |  |
|  | Start Timer for 30 minutes, Fly racetrack |  |  |
|  | Record MAP, OAT, and everything else |  |  |

NOTES

| Check | Action |
| --- | --- |
|  | # LANDING |
|  | At end of 30 minutes, switch tanks & descend to land |
|  | Use checklists |
|  | Fly pattern at 85 MPH |
|  | Taxi back and “Smile” |
|  | # POST FLIGHT |
|  | Measure fuel in each tank & fill |
|  | Record fuel burned & compute consumption rate |
|  | Left tank fuel added (burned) x 2 = GPH @ 2400 RPM (XX% power) @ 11,500 feet. |
|  | Right tank fuel added (burned) = fuel required to climb/descend to 11,500 feet. |
|  | Prepare corrective action list |
|  | Record fuel and oil consumption |
|  | - Repeat at this altitude using 2500, 2600 and 2700 RPM |

NOTES

| Flight #: 13 – MAGNETIC COMPASS CALIBRATION |  |  |  |
| --- | --- | --- | --- |
| Pilot: |  | Date: | Time: |
| Objectives: - Adjust compass & calibrate compass card |  |  |  |
| Check | Action |  |  |
|  | # PRE-FLIGHT |  |  |
|  | Pick a very calm morning |  |  |
|  | # DEPARTURE |  |  |
|  | Do not use flaps |  |  |
|  | Climb out at 110 MPH |  |  |
|  | Climb to 2000’ MSL and level off |  |  |
|  | # CRUISE |  |  |
|  | Limit prop RPM to 2400 |  |  |
|  | Check engine gauges especially CHT and Oil Temp |  |  |
|  | Trim hands off level flight |  |  |

NOTES

| Check | Action |
| --- | --- |
|  | CRUISE |
|  | # Fly North along a N/S Road |
|  | Check GPS compass heading |
|  | Adjust EFIS compass to read 360 degrees |
|  | Fly South, adjust EFIS compass |
|  | Fly East, adjust EFIS compass |
|  | # Fly West, adjust EFIS compass |
|  | # LANDING |
|  | Use checklists |
|  | Fly pattern at 85 MPH |
|  | Taxi back and “Smile” |
|  | # POST FLIGHT |
|  | Record errors on compass card |
|  | Prepare corrective action list |
|  | Record fuel and oil consumption |

NOTES

| Flight #: 14 – NIGHT OPERATION |  |  |  |
| --- | --- | --- | --- |
| Pilot: |  | Date: | Time: |
| Objectives: - Ensure lighting, etc. is safe for night flights |  |  |  |
| Check | Action |  |  |
|  | # PRE-FLIGHT |  |  |
|  | All 40 flight hours must have been accomplished before night operations can be executed with N57EN |  |  |
|  | Sit in aircraft in the dark |  |  |
|  | # Ensure flashlight is handy & has fresh batteries |  |  |
|  | # Set interior light brightness |  |  |
|  | # Are all instruments illuminated? |  |  |
|  | # Turn on all electricity. Does it exceed 80% (28 amps) of alternator capacity? |  |  |
|  | # Taxi aircraft at least 1/2 hour at night before flying at night (watch CHT & Oil Temps) |  |  |
|  | # FLIGHT #1 |  |  |
|  | # Start test at dusk w/ a little light remaining |  |  |
|  | # Perform at least 3 takeoffs & landings |  |  |
|  | # END TEST #1 |  |  |
|  | # Post flight Questions: |  |  |
|  | - Are taxi & landing lights effective? |  |  |
|  | - Is the strobe reflecting off anything? |  |  |
|  | - Are interior lights effective? |  |  |
|  | # Do lights reflect off canopy? |  |  |
|  | - Do any lights cause radio interference? |  |  |
|  | # POST FLIGHT |  |  |
|  | Prepare corrective action list |  |  |
|  | Record fuel and oil consumption |  |  |

NOTES